CASE STUDY -2

NAME – LAWANYA

EN. NO.- BT23CME042

- Introduction

The proposed product is a next-generation electric bicycle combining eco-friendly transportation with advanced smart features, designed to meet the growing needs of urban commuters, students, and environmentally conscious riders.
This E-Cycle (Smart Pedalling with Electric Drive) integrates multiple innovations:

- Torque & cadence sensors for smooth pedalling assistance and throttle mode for effortless riding

- The e-bike now supports tri-mode charging—plug-in, solar, and pedal-to-charge regeneration—improving energy independence and adding 3–7 km per 30 minutes of dedicated charging pedalling under typical conditions

- Built-in GPS tracker for navigation and theft protection

- Hydraulic disc brakes and puncture-resistant tyres for enhanced safety and reliability

- Detachable storage compartment for daily carry needs.

- Speedometer to measure speed.

The aim is to provide an affordable, lightweight, high-performance e-bike that merges convenience, safety, and sustainability.

2. Customer Statements        Design Interpretation      Product feature

| Customer Statements | Design Interpretation | Product feature |
| --- | --- | --- |
| I want an e-bike that is lightweight but sturdy. It should be easy to park and store. It should be durable for at least 5 years. | Provide strong yet lightweight frame with high service life. weatherproof components, warranty coverage. | Lightweight alloy frame, ≤ 20 kg, lifespan ≥ 5 years. |
| It should allow me to ride smoothly without much effort. I want both pedal assist and throttle mode. I want custom speed limits for safety I prefer a silent motor. | Enable hybrid power modes with intelligent sensors. Dual power drive system: pedal assist + throttle control. Programmable electronic speed controller (ESC). Low-noise brushless DC motor. | Torque + cadence sensors, pedal assist + throttle, programmable controller, brushless silent motor. |
| I need to charge it even without access to electricity. I want to charge the battery while pedalling or when stationary. I want a solar charging option for outdoor trips. I need fast charging when plugged in. I want the battery to last long on a single charge. It should not overheat in the sun. | Provide energy independence through solar, fast charging, and thermal protection. Enable bidirectional energy flow so the motor/controller can operate as a generator during selected modes (regen/braking/stand charging). | Dual charging via solar and plug-in with fast charging ≤ 3 hrs, supporting 100–150 km range with heat-resistant design. Added pedal-to-charge mode enables on-ride regeneration (e.g., descents or resistance mode) and stationary charging using a trainer/center-stand. Implemented through a regen-capable controller, BMS with current handling, and a direct-drive hub or regen-enabled mid-drive motor. |
| It should have good braking for safety. I need tyres that resist punctures. I want safety lights integrated. | Focus on user safety in braking, traction, and visibility. | Hydraulic disc brakes with short stopping distance. Puncture-resistant, durable tyres. LED headlight and rear safety lights integrated.. |
| I want to track my bike in case it gets stolen. I want built-in GPS navigation. | Built-in GPS module with anti-theft tracking. Integrated GPS navigation system with mobile app interface. | GPS tracker + mobile app integration. |
| It should carry my backpack or groceries. I prefer detachable storage. | Provide convenient load-carrying. Modular storage unit with quick-release mechanism. | Modular detachable storage (5–10 L). |
| It should have low running costs. I want minimal maintenance needs. I want parts to be easily replaceable I want a warranty on the product. | Provide Energy-efficient motor and maintenance-friendly design. Modular parts, sealed bearings, and durable drivetrain. Standardized, modular spare parts system. Comprehensive warranty on frame, motor, and battery. | Modular replaceable parts, energy-efficient system, product warranty. |
| I want it to be comfortable for long rides. I want it to be suitable for both city and campus rides. | Provide Ergonomic seat, shock absorbers, and optimized geometry. All-terrain tyres, adaptable speed settings, compact size. | Ergonomic saddle, suspension, multipurpose tyres. |
| I need an easy-to-read display | Ensure clear ride information | Digital LCD/LED display with speed, battery, GPS. |
| I want it to be stylish and modern looking. I want an affordable price point. It should be available in multiple colours. | Market-focused design aesthetics. | Sleek modern design with aerodynamic frame. Cost-effective production, retail price ≤ ₹50,000. Multi-colour variants with scratch-resistant coating. |

3. Benchmarking Table: Customer Needs vs. Competitors

| Customer Need | Hero Lectro EHX20 | Merida eBig.Nine 400 | RadCity 5 Plus | Giant Explore E+2 | Our Smart E-Bike |
| --- | --- | --- | --- | --- | --- |
| Lightweight, durable | Medium | Medium | Medium | High | High (≤20 kg alloy frame) |
| Pedal-to-charge / regeneration | No | No | Partial | No | Yes |
| Pedal assist + throttle | Assist only | Yes | Yes | Yes | Full hybrid (assist + throttle) |
| Solar charging | No | No | No | No | Yes (dual charging) |
| Fast charging | No | Partial | No | Partial | Yes (≤3 hrs) |
| GPS tracking | No | No | No | No | Yes |
| Braking safety | Mechanical. | Hydraulic | Mechanical | Hydraulic | Hydraulic |
| Puncture-resistant tyres | No | No | Yes | No | Yes |
| Storage | Basic | None | None | None | Detachable 5–10 L |
| Comfort for long rides | Average | Good | Good | Good | Good |
| Affordability | ₹1.3 L | ₹2.8 L | ₹1.65 L | ₹2.25 L | ₹0.5–1.0 L |

4. Benchmarking on Metrics

| Metric | Hero Lectro EHX20 | Merida eBig.Nine 400 | RadCity 5 Plus | Giant Explore E+2 | Our Smart E-Bike |
| --- | --- | --- | --- | --- | --- |
| Battery Range (km) | 60–70 | 100+ | 70–80 | 100+ | 100-150 |
| Charging Time (hrs) | 5–6 | 4–5 | 6 | 4–5 | 3-4 |
| Motor Power (W) | 250 | 350 | 250 | 350 | 350 |
| Regen availability | No. | No. | No. | No. | Yes (pedal + brake + stand) |
| Net pedal-charge power | No. | No. | No. | No. | 100–150 W |
| Peak regen current to battery | No. | No. | No. | No. | 5–8 A at 36–48 V |
| Added range from 30 min pedal-charge | No | No | No | No | 3–7 km |
| GPS | No | No | No | No | Yes |
| Solar Charging | No | No | No | No | 30–40 km/day |
| Storage (L) | Basic | None | None | None | Detachable 5–10 L |
| Price | ₹1.3 L | ₹2.8 L | ₹1.65 L | ₹2.25 L | ₹0.5–1.0 L |

From benchmarking we observe that current e-bikes lack:

- Solar charging → essential for energy independence.

- GPS tracking → critical for navigation and theft protection.

- Detachable storage → important for commuting convenience.

- Pedal-to-charge regeneration → enables on-ride and stationary energy recovery for extended range.

These gaps emphasize the opportunity to design a smart e-bike that ensures self-sufficiency, safety, and convenience for urban riders.

5. Problem Clarification & Sub-Problems

| Sub-Problems | Marginal Specification | Ideal Specification |
| --- | --- | --- |
| Power | 80 km range; plug-in charging only | 150 km range; solar + plug-in dual charging |
| Drive | 250 W motor; pedal assist only | 350 W silent motor; assist + throttle; torque sensors |
| Safety | Mechanical brakes; basic tyres | Hydraulic brakes; puncture-proof tyres; lights |
| Smart | Phone GPS only | Built-in GPS with anti-theft; mobile app |
| Pedal-to-charge (regen) | Brake-only regen; ≤ 80 W avg; simple on/off | Pedal + brake + stand modes; 100–150 W continuous, 200–250 W peak; efficiency ≥ 75%; user-selectable resistance; BMS with dynamic regen current limit |
| Utility | Basic carrier | Modular detachable storage (5–10 L) |
| Comfort | Average saddle | Ergonomic saddle + suspension |
| Cost | ≥ ₹1.3 L | ≤ ₹1.0 L |

6. Need–Metric Matri

|  | Frame weight ≤20kg, | long life | cadence sensor | cadence sensor | Hybrid drive (assist + throttle) | Pedal-charge power | Fast charging | GPS accuracy | Puncture-resistant tyres | Warranty | Colour | Display visibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lightweight but sturdy |  |  |  |  |  |  |  |  |  |  |  |  |
| Pedal assist + throttle mode |  |  |  |  |  |  |  |  |  |  |  |  |
| Smooth riding with less effort |  |  |  |  |  |  |  |  |  |  |  |  |
| Charge without electricity |  |  |  |  |  |  |  |  |  |  |  |  |
| Fast charging + Long battery life |  |  |  |  |  |  |  |  |  |  |  |  |
| GPS tracking & navigation |  |  |  |  |  |  |  |  |  |  |  |  |
| Puncture-resistant tyres |  |  |  |  |  |  |  |  |  |  |  |  |
| Minimal maintenance & Stylish |  |  |  |  |  |  |  |  |  |  |  |  |
| Easy-to-read display |  |  |  |  |  |  |  |  |  |  |  |  |

7. Need–Metric Analysis

| Customer Need | Engineering Metric | Marginal Value | Ideal Value |
| --- | --- | --- | --- |
| Long battery life | Range (km/charge) | ≥ 80 | ≥ 150 |
| Solar charging | Solar output (km/day) | ≥ 10 | ≥ 40 |
| Smooth riding | Sensor response (ms) | ≤ 200 | ≤ 100 |
| Charge while pedalling / stationary | Net pedal-charge power (W); Energy in 30 min (Wh); Added range per 30 min (km) | ≤30% | 100–150 W; 50–70 Wh; 3–7 km; ≥ 75% |
| Safety braking | Stopping distance (m @25 km/h) | ≤ 5 | ≤ 3 |
| Theft protection | GPS accuracy (m) | ≤ 10 | ≤ 5 |
| Storage capacity | Volume (L) | ≥ 3 | ≥ 10 |
| Comfort | Ergonomic rating | Standard | High |
| Display | Visibility rating (1–5) | ≥ 3 | ≥ 5 |
| Affordability | Price (₹) | ≤ 1.3 L | ≤ 1.0 L |

8. Target Specifications per Subsystem

- Power System
• 100–150 km range

• ≤ 3 hrs fast charging

• Solar backup 30–40 km/day

•Battery: 36–48 V Li-ion; BMS supports regen current 5–8 A

•Solar + Plug-in + Pedal-to-charge tri-charging architectur

- Drive System
• 350 W silent motor

• Torque + cadence sensors

• Pedal assist + throttle

•Regen-capable motor

- Safety System
• Hydraulic disc brakes (≤ 3 m stop)

• Puncture-resistant tyres

• LED front + rear lights

•Auto-regen cut-back at high SOC/high temp; fail-safe when battery full

- Smart System
•  GPS tracker with ±5 m accuracy.

•  Anti-theft lock integrated with mobile app.

• Pedal-to-charge monitoring via display/app to show regenerated energy and added range.

- Utility System
• Detachable storage (5–10 L)
• Weight ≤ 20 kg

- Aesthetics & Market Appeal
• Sleek, modern design
• Multiple colour options
• Multiple type of display screen

9. External & Internal Search

External Search:

- Competitor benchmarking (Hero Lectro, Merida, RadCity, Giant).

- Supplier research: solar panels, GPS modules, regen lithium-ion batteries.

- Customer surveys: urban commuters, students, delivery riders.

- Literature: global e-bike adoption trends (CAGR ~10%).

Internal Search:

- Brainstorming and modelling of frame + solar integration + pedal to charge .

- Existing know-how in alloy frame design and lightweight systems.

- Prototyping with modular storage, display and GPS app.

- Supplier collaboration for motor, tyres, and brakes.